Soldering Guide

General Tips

- Prep the Iron: Heat your iron to roughly 350 °C (650 °F - 750 °F). Make sure the tip of the iron is clean for optimal heat transfer. Wipe the tip on damp brass/sponge.

- Apply heat: Touch the tip of the iron to both the component lead and the copper pad at the same time. Wait 2 to 3 seconds for the metal to heat up.

- Feed Solder: Touch your solder wire to the heated joint (not the iron itself). Let the pad and lead melt the solder so it flows evenly.

- Cool: Remove the solder and iron, then let the joint cool completely without moving the component.

Note: Heat the joint, not the solder. Don’t use too much solder, as it can cause short circuits and other issues.

SmartTrap Soldering

Figure 0. Seeed Studio pin layout.

- Insert the seeed studio into the prototype board and solder the pins. Make sure to leave at least 2 pins on each side of the seeed microcontroller (Figure 1).

Figure 1. Seeed studio inserted into a prototype board

- Insert wires to Jump the ground and 3V connections. You can bend the wires behind the board to secure the wires before soldering. Make sure to leave at least 2 or three lines between jump destinations (Figure 2). Extend the ground for three lines, and 2 lines for the 3V connection (Figure 2C).

Figure 2. Solder power extensions for the board.

- Solder Male header pins to the board. This will make it easy to connect the other components securely to the prototype board. Insert the header pins into the board (Figure 3A). You can use tape to keep the pins in place before soldering (Figure 3B). This will prevent the pins from falling off when you flip the board for soldering. You can also tape the board to the bench to secure it for soldering (Figure 3C).

Figure 3. Solder Male header pins to the prototype. This will make it easy to connect 	components to the board securely.

- Solder all other Male header pins for all connections to the board. Use the same procedure as in step 3. Insert the pins, tape the pins, flip the board, and solder. Do this until all header pins are soldered to the prototype board. The final board should have 2 pins for D0, D1, D4, D5, D6, and D7.  D3 and 5V only need one pin. Finally, the ground should have 11 pins, and 3V should have 7 pins (Figure 4C).

Figure 4.  Solder all other connection pins to the board.

Additional Resources

- https://www.ifixit.com/News/97850/soldering-simplified-a-newbies-guide-to-through-hole-soldering