# SmartTrap v1.0

Field Deployment & Maintenance Guide

# 1. Field Deployment

## Site Selection

- Place trap at field edge or within crop canopy

- Avoid direct sunlight on electronics

- Ensure IR sensor path is clear

- Position soil sensors in representative soil

## Installation Checklist

- ☐ SD card formatted FAT32

- ☐ Power bank fully charged

- ☐ RTC time verified

- ☐ Fresh pheromone lure

- ☐ IR sensor aligned

- ☐ Enclosure sealed

## Expected Battery Life

| Power Bank | Capacity | Runtime |
| --- | --- | --- |
| Small | 10,000 mAh | 3-4 weeks |
| Medium | 20,000 mAh | 6-8 weeks |

# 2. Maintenance Schedule

## Weekly Tasks

- Check power bank level

- Download data via BLE

- Clear trap of insects

- Verify IR sensor alignment

## Bi-Weekly Tasks

- Replace pheromone lure (2-4 weeks)

- Clean camera lens

- Inspect for moisture

## Monthly Tasks

- Full data export and analysis

- Reset device (clear old recordings)

- Verify RTC accuracy

# 3. Data Collection

## Files to Download

The system creates two separate CSV files for better analysis.

| File | Contents |
| --- | --- |
| /logs/environment.csv | Periodic readings: timestamp, air_temp, humidity, soil_temp, soil_moisture |
| /logs/detections.csv | Detection events: timestamp, detection_num, env data, video/audio paths |
| /events/YYYYMMDD/ | Video (.avi) and audio (.wav) recordings by date |

## Data Collection via BLE

- Open SmartTrap_v1.0_Client.html in Chrome

- Click "Connect" and select device

- Enter password (default: smart2025)

- Navigate to /logs/ folder

- Download environment.csv and detections.csv

- Navigate to /events/ for video/audio files

## Web Client Status Dashboard

The web client provides comprehensive device monitoring for field maintenance:

Device Info: Device name, firmware version, uptime, RTC time, schedule, active/sleep mode

Component Status: LCD, RTC, DHT11, DS18B20, Camera, Microphone, SD Card, BLE, IR Sensor (OK/FAIL indicators)

Storage Info: Visual progress bar showing SD card usage (total, used, free)

Memory Info: Free heap, PSRAM, minimum heap (helps detect issues)

Environmental Sensors: Air temp, humidity, soil temp, soil moisture with live readings

Power Info: Battery percentage, voltage, charging status (when available)

## Auto-Refresh Feature

The client includes auto-refresh to continuously monitor device status:

- Off - Manual refresh only

- 10 seconds - Active debugging

- 30 seconds - Close monitoring

- 1 minute - Normal field use (default)

- 5 minutes - Extended monitoring

- 10/30 minutes - Long-term observation

A countdown timer shows when the next refresh will occur.

## Data Offloading via USB Drive Mode

To download data via USB, press the BUTTON during startup:

- Plug USB-C cable into your computer

- Press BUTTON within 10 seconds when you see the countdown

- SD card appears as a removable USB drive

- Copy /logs/ and /events/ folders to your computer

- Safely eject the drive and unplug

Note: If you don't press the button, the device enters Normal Mode (monitoring). This is intentional for field deployment with power banks!

## Data Offloading via SD Card Reader (Alternative)

For faster bulk transfers, remove the SD card and use a USB card reader:

- Power off the device

- Gently remove the microSD card from the expansion board slot

- Insert into a USB microSD card reader

- Copy /logs/ and /events/ folders to your computer

- Re-insert card and power on device

Note: This is the fastest method for downloading large video/audio files. BLE transfer is best for quick CSV downloads in the field.

# 4. Common Field Issues

| Issue | Solution |
| --- | --- |
| False detections | Adjust IR alignment, shield from wind |
| Missed detections | Clean IR sensor, verify LED working |
| Moisture damage | Improve sealing, add desiccant |
| SD card full | Download data, then RESET |

# 5. Support

Project PI: Dr. Edward Idun Amoah (Penn State)

Ghana PI: Dr. Kofi Frimpong-Anin (CSIR-CRI)