# SmartTrap v1.0 - Quick Reference

## Pin Connections

D0 (GPIO1) → Soil Moisture AO

D1 (GPIO2) → DS18B20 DATA + 4.7kΩ pull-up to 3.3V

D2 (GPIO3) → DHT11 DATA

D3 (GPIO4) → Button → GND + 10kΩ pull-up to 3.3V

D4 (GPIO5) → I2C SDA (LCD + RTC)

D5 (GPIO6) → I2C SCL (LCD + RTC)

D6 (GPIO43) → IR LED (+) via 100Ω

D7 (GPIO44) → IR Receiver OUT + 10kΩ pull-down to GND

D8-D10 → RESERVED (SD Card)

## Button Controls

During startup countdown: Press BTN → USB Drive Mode

Short press (<1s): Toggle LCD backlight

Long press (5s): Toggle BLE on/off

Press during sleep: Wake device

## Default Settings

Password: smart2025

Active hours: 8 PM - 6 AM

Recording: 10s @ 15 FPS

Env logging: Every 1 minute

BLE Name: SmartTrap_001

## Log Files

/logs/environment.csv - Periodic env data

/logs/detections.csv - Moth events + env

/events/YYYYMMDD/ - Video/audio files

## Startup Modes

Normal Mode (default): Don't press button → Monitoring starts

USB Drive Mode: Press BTN during countdown → Data transfer

BLE Data Transfer: Use web client, authenticate, download files

## Env Logging Intervals

60000 = 1 min | 300000 = 5 min | 3600000 = 1 hour

## BLE Commands (Public)

STATUS - Device info, time, schedule, detections

DIAG - Component status, memory, storage, battery

SENSORS - Environmental sensor readings

DETECTIONS - Current detection count

RECORD - Trigger manual recording

## BLE Commands (Protected)

AUTH:password - Authenticate for data access

LIST:/path - List directory contents

GET:/path/file - Download file

DELETE:/path/file - Delete file

RESET - Factory reset (deletes all data)

## Quick Fixes

No serial: Enable USB CDC On Boot

Upload fail: BOOT → RESET → Release

SD error: FAT32, ≤32GB

Penn State & CSIR-CRI Collaboration