# SmartTrap v1.0

Firmware Installation & Configuration Guide

# 1. Prerequisites

## Required Software

- Arduino IDE 2.0+ (download from arduino.cc)

- ESP32 Board Package (version 2.0.8 or higher)

- Chrome browser (for Web Bluetooth client)

## Required Libraries

Install via Arduino Library Manager:

- LiquidCrystal_I2C by Frank de Brabander

- RTClib by Adafruit

- DHT sensor library by Adafruit

- OneWire by Paul Stoffregen

- DallasTemperature by Miles Burton

# 2. Arduino IDE Setup

- Open File → Preferences

- Add to Additional Boards Manager URLs:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Tools → Board → Boards Manager → Search "esp32"

- Install "esp32 by Espressif Systems" (latest version)

- Tools → Board → esp32 → Select "ESP32S3 Dev Module"

- CRITICAL: Set "USB CDC On Boot" → "Enabled"

- Set "PSRAM" → "OPI PSRAM"

## Port Selection

Select the correct port in Tools → Port:

- Mac: /dev/cu.usbmodem101 (or /dev/cu.usbmodemXXXX)

- Windows: COM3, COM4, or higher (COM1/COM2 are reserved)

- Linux: /dev/ttyACM0 or /dev/ttyUSB0

Tip: Unplug the device, note the ports, plug it back in - the new port is your XIAO.

# 3. Configuration Options

Edit these values at the top of the firmware file:

| Setting | Default | Description |
| --- | --- | --- |
| DEVICE_NAME | "SmartTrap_001" | Bluetooth device name |
| AUTH_PASSWORD | "smart2025" | Password for file access |
| ENV_LOG_INTERVAL_MS | 60000 (1 min) | Env logging interval |
| ENABLE_SCHEDULED_SLEEP | true | Enable sleep schedule |
| USB_MSC_ENABLED | true | USB drive auto-mode |
| ACTIVE_START_HOUR | 20 (8 PM) | Hour to start monitoring |
| ACTIVE_END_HOUR | 6 (6 AM) | Hour to stop monitoring |
| SLEEP_CHECK_INTERVAL | 300000 (5 min) | Check frequency to go to sleep |
| WAKE_CHECK_INTERVAL | 1800000 (30 min) | Check frequency to wake up |
| VIDEO_FPS | 15 | Video frame rate |
| RECORDING_DURATION | 10000 (10s) | Recording length in ms |

## Environmental Logging Interval Examples

- 60000 = 1 minute (default)

- 300000 = 5 minutes

- 600000 = 10 minutes

- 3600000 = 1 hour

# 4. Data Logging

The system uses separate logging for environmental data and moth detections, allowing better correlation analysis.

## Two CSV Log Files

| File | Purpose |
| --- | --- |
| environment.csv | Periodic environmental readings (configurable interval) |
| detections.csv | Moth detection events with env conditions at time of detection |

## environment.csv Format

Logged every minute (or configured interval):

timestamp, air_temp, humidity, soil_temp, soil_moisture

Example: 2024-12-11 20:01:00, 24.5, 65.0, 18.2, 2150

## detections.csv Format

Logged at each moth detection event:

timestamp, detection_num, air_temp, humidity, soil_temp, soil_moisture, video_file, audio_file

## Benefits of Separate Logging

- Continuous environmental monitoring for trend analysis

- Correlate moth activity with temperature, humidity, and soil conditions

- Smaller detection log (only events, not continuous data)

- Better data for degree-day models and outbreak prediction

# 5. BLE Diagnostic Commands

The device responds to these BLE commands for monitoring and maintenance:

## STATUS Command

Returns: v (firmware), name, det (detections), auth, time, sched, active, uptime

## DIAG Command

Returns component status for: lcd, rtc, dht, ds18, cam, mic, sd, ble, ir

Also returns MEMORY (heap, psram, minHeap), SDINFO (total, used, free, pct), BATTERY (pct, voltage, charging)

## SENSORS Command

Returns: airT, hum, soilT, soilM, time, dhtOK, dsOK

## Protected Commands (require AUTH:password first)

LIST:/path - List directory | GET:/path/file - Download | DELETE:/path/file - Remove | RESET - Factory reset

# 6. Button Controls

| Action | Function |
| --- | --- |
| During startup countdown | Press BTN → USB Drive Mode (data transfer) |
| No button press | Normal Mode (monitoring/programming) - DEFAULT |
| Short press (<1s) | Toggle LCD backlight on/off |
| Long press (5+ seconds) | Toggle Bluetooth on/off |
| Press during sleep | Wake device from deep sleep |

# 7. Troubleshooting

| Problem | Solution |
| --- | --- |
| No Serial output | Enable "USB CDC On Boot" in Tools |
| Upload fails | Hold BOOT, press RESET, release, upload |
| SD card not detected | FAT32 format, ≤32GB, firmly seated |
| DS18B20 reads -999 | Check 4.7kΩ pull-up resistor |
| BLE not visible | Long press button (5s) to enable BLE |