# SmartTrap v1.0

Hardware Assembly Guide

# 1. System Overview

The SmartTrap is an automated monitoring system designed to detect and count Fall Armyworm moths entering pheromone traps. It uses dual-pair infrared beam-break detection (4 LEDs + 4 receivers on D6 and D0) in a star pattern for maximum sensitivity, records JPEG image evidence, and provides wireless data access via Bluetooth.

## Key Features

- Automatic moth detection via 4 IR beam-break sensors (2 pairs on D6 + D0, star pattern)

- 10-frame JPEG burst capture per detection event

- Environmental sensors removed in this build to free D0 for IR LED pair #2

- Detection logging with CSV format (environmental sensors not installed)

- SD card data logging with CSV format

- BLE removed in firmware v1.5 (frees memory for camera)

- Scheduled sleep mode for extended battery life

- Password protection for data security

# 2. Required Components

| Component | Specification | Qty | Cost |
| --- | --- | --- | --- |
| Microcontroller | XIAO ESP32S3 Sense | 1 | $15 |
| Expansion Board | XIAO Sense Camera/Mic Board | 1 | $10 |
| LCD Display | 16x2 I2C LCD (0x27 or 0x3F) | 1 | $5 |
| Real-Time Clock | DS3231 RTC Module | 1 | $8 |
| IR LED | 940nm IR Emitter (5mm), 2 pairs | 4 | $1 |
| IR Receiver | TSOP38238 IR Receiver (38kHz, 940nm) | 4 | $1 |
| DHT11 (REMOVED) | Not installed — pin D2 free | 1 | $3 |
| DS18B20 (REMOVED) | Not installed — pin D1 free | 1 | $5 |
| Soil Moisture (REMOVED) | Not installed — pin D0 used for IR | 1 | $3 |
| Push Button | 4-pin Tactile Switch | 1 | $1 |
| SD Card | MicroSD FAT32 (≤32GB) | 1 | $8 |
| Resistors | 2.2kΩ (x2 for I2C) | 1 each | $0.30 |
| Power Bank | USB 10,000-20,000mAh | 1 | $15-25 |
| TOTAL |  |  | ~$75-90 |

# 3. Pin Configuration

| XIAO Pin | GPIO | Connection | Notes |
| --- | --- | --- | --- |
| D0 | GPIO1 | IR LEDs #3+#4 | 38kHz PWM output (parallel) |
| D1 | GPIO2 | Free (future sensor) | Available |
| D2 | GPIO3 | Free (future sensor) | Available |
| D3 | GPIO4 | Button → GND | INPUT_PULLUP (no external resistor) |
| D4 | GPIO5 | I2C SDA (LCD + RTC) | Shared I2C bus |
| D5 | GPIO6 | I2C SCL (LCD + RTC) | Shared I2C bus |
| D6 | GPIO43 | IR LEDs #1+#2 | 38kHz PWM output (parallel) |
| D7 | GPIO44 | All 4 IR Receivers OUT (parallel) | INPUT_PULLUP, no external resistor |
| D8-D10 | GPIO7,8,9 | Free (future SIM7670G cellular) | Reserved for cellular |

IMPORTANT: Pins D8, D9, D10 (GPIO 7, 8, 9) are reserved for future SIM7670G cellular module. SD card uses internal B2B connector's SD card slot.

# 4. Wiring Connections

## Power Distribution

- 3.3V → All 4 IR Receivers, RTC, LCD

- 5V → LCD if needed (some work on 3.3V)

- GND → All sensor GND pins (common ground)

## IR Beam-Break Sensor

- IR LEDs #1+#2: Anode (+) → D6 (GPIO43), Cathode (-) → GND. IR LEDs #3+#4: Anode (+) → D0 (GPIO1), Cathode (-) → GND.

- All 4 IR Receivers: VCC → 3.3V, GND → GND, all 4 OUT pins connect to D7 in parallel (logical OR). Any beam break triggers the interrupt.

- No external pull-down needed. Use INPUT_PULLUP in firmware. Position beam pairs in a star pattern across the funnel opening for maximum coverage.

- Each LED-receiver pair faces each other, 2-10cm apart. Arrange 4 beams at ~45° intervals across the funnel.

## IR LED Pairs Wiring Detail

- D6 pair: LED #1 anode → D6, LED #2 anode → D6. Both cathodes → GND.

- D0 pair: LED #3 anode → D0, LED #4 anode → D0. Both cathodes → GND. Both pins output 38kHz PWM simultaneously.

## I2C Bus (LCD + RTC)

- LCD SDA + RTC SDA → D4 (GPIO5)

- LCD SCL + RTC SCL → D5 (GPIO6)

# 5. Verification Checklist

- ☐ SD card inserted and formatted FAT32

- ☐ Expansion board attached (audible click)

- ☐ No shorts between 3.3V and GND

- ☐ 4 IR LEDs connected (no series resistor)

- ☐ LEDs #1+#2 wired to D6, LEDs #3+#4 wired to D0

- ☐ All 4 IR Receiver OUT pins connected to D7 (parallel, INPUT_PULLUP)

- ☐ Button wired D3 → GND (INPUT_PULLUP, no external resistor needed)

- ☐ D8-D10 pins free for future cellular module

- ☐ 4 IR beam pairs arranged in star pattern across funnel (2-10cm gap each)

Tip: Use a phone camera to verify the IR LED is working - it will appear as a faint purple glow.